Ammonia

The Haber Process

Simulation: edtechsims.com/haber-process

Name: ____________________  Class: ______  Date: _______

# Part A: The Reaction

N₂(g)  +  3H₂(g)   ⇌   2NH₃(g)          forward reaction is exothermic

A1  Complete the sentences.	[6]

The nitrogen used in the Haber process comes from ______________ (by fractional distillation of liquid air). The hydrogen comes from ______________ or the cracking of hydrocarbons.

The symbol ⇌ shows that the reaction is ______________. In a closed system it reaches ______________, where the forward and reverse reactions happen at the ______________.

Most of the ammonia made is used to make ______________, such as ammonium nitrate and ammonium sulfate.

# Part B: Effect of Temperature

On the sim: Click on ‘A Microscopic View’. Observe the pressure at 200 atm. Unselect the iron catalyst checkbox. Select different temperatures and record the yields of ammonia.

B1  Complete Table 1.	[3]

| Temperature / °C | Yield of NH₃ / % | Speed of particles (slow / medium / fast) |
| --- | --- | --- |
| 350 |  |  |
| 450 |  |  |
| 550 |  |  |

Table 1

B2  Describe and explain the effect of increasing the temperature on the yield of ammonia.	[2]

B3  Why is a very low temperature, such as 200 °C, not used even though it gives a higher yield?	[2]

# Part C: Effect of Pressure

On the sim: Click on ‘A Microscopic View’. Set the temperature to 450 °C. Unselect the iron catalyst checkbox. Change the pressure and record the yield of ammonia.

C1  Complete Table 2.	[4]

| Pressure / atm | Yield of NH₃ / % |
| --- | --- |
| 100 |  |
| 200 |  |
| 300 |  |
| 400 |  |

Table 2

C2  How many molecules of gas are on each side of the equation?	[1]

left side: ........      right side: ........

C3  Use your answer to C2 to explain the effect of increasing the pressure on the yield.	[2]

C4  Suggest two reasons why the pressure is not raised to 1000 atm.	[2]

# Part D: Effect of the Iron Catalyst

On the sim: Click on ‘A Microscopic View’. Set 450 °C and 200 atm. Turn the iron catalyst on and off. Compare the yield and how quickly the particles react.

D1  Complete Table 3.	[2]

| Iron catalyst | Yield of NH₃ / % | Rate at which equilibrium is reached |
| --- | --- | --- |
| Off |  | Slower |
| On |  | Faster |

Table 3

D2  Explain why the catalyst does not change the yield but is still used.	[2]

# Part E: The Compromise Conditions

E1  Click on ‘Plant Manager’. Complete Table 4 with the conditions used in industry and the reason for each.	[6]

| Condition | Value | Reason |
| --- | --- | --- |
| Temperature |  |  |
| Pressure |  |  |
| Catalyst |  |  |

Table 4

E2  Only about 15% of the nitrogen and hydrogen is converted to ammonia each time the gases pass over the catalyst. Explain how the factory still uses almost all the raw materials.	[2]

# Part F: Practice Questions

F1  Fig. 1 shows the yield of ammonia at different temperatures and pressures (approximate data).

Fig. 1

(a)  State the yield of ammonia at 450 °C and 200 atm.	[1]

yield = ............ %

(b)  Use Fig. 1 to describe the effect of temperature on the yield at 300 atm.	[2]

(c)  Suggest why a plant might choose 450 °C and 200 atm instead of 350 °C and 400 atm, even though the yield is lower.	[2]

F2  Calculate the maximum mass of ammonia that can be made from 56 tonnes of nitrogen. (Ar: N = 14, H = 1)	[3]

mass = ............ tonnes

F3  Describe a test for ammonia gas.	[2]